# CODE BOOKLET

# Modelling and Analysis of Cellular Steel Beams

ACADEMIC SESSION: 2024/2025

STUDENT: FARZAD KHAN CHOWDHURY (A21MJ0138)

PROGRAM: Bachelor of Computer Science (Software Engineering ) with Honours

SUPERVISOR: PROF. DR. HALINAWATI HIROL

CO-SUPERVISOR:

SUMMARY:

This document holds all the coding for the final year project Modelling and Analysis of Cellular Steel Beams which include all functions, automation. The system solves key challenges in analyzing cellular steel beams. It eliminates the need for manual modeling in ABAQUS by automating geometry creation, loading, meshing, and result extraction. Users can quickly test multiple design configurations without repeating steps. The built-in GUI makes the system accessible to users with little or no FEA experience. It also removes the need for manual interpretation of results by automatically evaluating stress and deflection against safety limits. Overall, the system saves time, reduces errors, and makes structural analysis faster and more efficient.

Table of Contents

- Project Hardware and Software List         3 - 4

- Modules List and Explanation                  5 - 6

- Code for Automation Function	       7 - 11

- Code for Gui function	                  12 -18

- Code for optimization with ga                 19 - 20

- User Manual                                             21 - 22

Project Hardware and Software List

| HARDWARE | VERSION | EXPLANATION OF USAGE |
| --- | --- | --- |
| ASUS TUF GAMING LAPTOP | Windows 11 Intel(R) Core(TM) i5-10300H CPU @ 2.50GHz 2.50 GHz | Used for coding, modelling, automation and documentation |

| SOFTWARE | VERSION | EXPLANATION OF USAGE |
| --- | --- | --- |
| Visual Studio Code | 2.1.2 | Used to develop and run experiments. |
| ABAQUS Student Edition | ABAQUS Student Edition 2021 | - Used for Finite Element Analysis (FEA) of the cellular steel beam models. - Handles geometry creation, boundary conditions, meshing, simulation, and result storage. - Scripts were run in noGUI mode for automation. |
| Python | 2.7 | - Required by ABAQUS for scripting and automation. - Used to create and run the simulation script (j.py) that controls model generation and analysis tasks. |
| Python | 3.14 | - Used for developing the standalone Graphical User Interface (GUI). - Enables real-time parameter input, simulation control, and result display for users. |
| Tkinter | Python GUI Library | - Built-in Python module used to design the GUI interface. - Allows users to enter hole diameter and spacing, run simulations, and view results. |
| CSV Module | Python | Used to write simulation outputs (stress, deflection, safety status) into a structured CSV file for later review. |
| Matplotlib |  | - Used to generate plots (e.g., stress vs. hole diameter, deflection trends). - Helps visualize performance data directly from the GUI or results file. |
| Terminal |  | - Used to launch the GUI (python ga.py) and execute scripts. - Ensures platform-independent execution (Windows/macOS/Linux) |

Module List and Explanation

| MODULE | FUNCTION | USERS |
| --- | --- | --- |
| j.py | ABAQUS automation script (Python 2.7) that reads user input, creates the cellular beam model, applies loads and boundary conditions, runs the simulation, and extracts results. | Student , Researchers, instructors, Professionals |
| Ga.py | Main GUI file (Python 3.x) that allows users to input hole diameter and spacing, trigger simulations, and display results such as max stress, deflection, and safety status | Student , Researchers, instructors, Professionals |
| beam_input.txt | Text file that stores input values (hole diameter and spacing) from the GUI for use by the simulation script. | Student , Researchers, instructors, Professionals |
| beam_results.csv | Output file that logs results of each simulation (max stress, deflection, safety verdict), used for display in the GUI or for graph generation. | Student , Researchers, instructors, Professionals |
| Optimization_with_ga.py | Uses a Genetic Algorithm (GA) to optimize beam design by generating and testing multiple hole diameter and spacing combinations. It runs j.py for each design, evaluates fitness based on deflection and stress, and evolves the best configuration through selection, crossover, and mutation.. | Student , Researchers, instructors, Professionals |

Code for J.py (Automation)

# -*- coding: utf-8 -*-

# ABAQUS Python script for modeling, simulating, and evaluating cellular steel beams

from abaqus import *

from abaqusConstants import *

from caeModules import *

import regionToolset

import mesh

import visualization

import odbAccess

import csv

import os

# --- Beam Properties ---

beam_length = 1000.0  # Total beam length in mm

initial_beam_height = 100.0  # Starting beam height (auto-adjusts if hole is too big)

beam_width = 50.0  # Width (depth) of the beam in mm

youngs_modulus = 210000.0  # Elastic modulus of steel (MPa)

poisson_ratio = 0.3  # Poisson’s ratio of steel

# Lower yield strength to allow more unsafe cases (for demonstration)

yield_strength = 200.0  # MPa

# Slightly relaxed deflection limit (serviceability criteria)

max_deflection = beam_length / 120.0  # Acceptable max deflection ≈ 8.33 mm

# --- Read user input from text file ---

input_file_path = 'beam_input.txt'

hole_diameter_list = []

hole_spacing_list = []

if os.path.exists(input_file_path):

with open(input_file_path, 'r') as f:

try:

d, s = map(float, f.read().strip().split(','))

hole_diameter_list = [d]

hole_spacing_list = [s]

print("[INFO] Using user input: D = {}, S = {}".format(d, s))

except:

print("[WARNING] Failed to parse user input. Using default values.")

hole_diameter_list = [60.0, 80.0, 100.0]

hole_spacing_list = [300.0, 400.0]

else:

hole_diameter_list = [60.0, 80.0, 100.0]

hole_spacing_list = [300.0, 400.0]

# --- Store results here ---

results = []

# --- Iterate through combinations of input values ---

for hole_diameter in hole_diameter_list:

for hole_spacing in hole_spacing_list:

job_name = 'Job_D{}_S{}'.format(int(hole_diameter), int(hole_spacing))

model_name = 'AutoBeamModel'

print("\n[INFO] Creating model: {} with job: {}".format(model_name, job_name))

print("[INFO] Hole Diameter: {}, Hole Spacing Param: {}".format(hole_diameter, hole_spacing))

# Adjust beam height if hole diameter is too large

beam_height = initial_beam_height

if hole_diameter > beam_height * 0.9:

beam_height = hole_diameter / 0.9

print("[INFO] Auto-adjusted beam height to {:.1f} mm".format(beam_height))

# Delete old model if exists

if model_name in mdb.models.keys():

del mdb.models[model_name]

model = mdb.Model(name=model_name)

# --- Sketch profile of the beam with holes ---

s = model.ConstrainedSketch(name='beam_profile', sheetSize=beam_length * 1.5)

s.rectangle(point1=(0.0, 0.0), point2=(beam_length, beam_height))

# Define hole positions based on spacing

num_holes = 3

spacing_between_holes = hole_spacing

hole_positions = [i * spacing_between_holes for i in range(1, num_holes + 1)]

# Check if holes are within beam bounds

invalid = False

for x in hole_positions:

if x - hole_diameter / 2.0 < 0 or x + hole_diameter / 2.0 > beam_length:

print("[ERROR] Hole at x={} with D={} would exceed beam boundary. Skipping.".format(x, hole_diameter))

invalid = True

break

if invalid:

continue

# Draw holes as circles

for x in hole_positions:

y = beam_height / 2.0

s.CircleByCenterPerimeter(center=(x, y), point1=(x + hole_diameter / 2.0, y))

# --- Create 3D beam part ---

try:

part = model.Part(name='CellularBeam', dimensionality=THREE_D, type=DEFORMABLE_BODY)

part.BaseSolidExtrude(sketch=s, depth=beam_width)

except:

print("[ERROR] Extrusion failed for D={} S={}".format(hole_diameter, hole_spacing))

continue

# --- Define Material and Section ---

material = model.Material(name='Steel')

material.Elastic(table=((youngs_modulus, poisson_ratio),))

section = model.HomogeneousSolidSection(name='BeamSection', material='Steel')

part.SectionAssignment(region=(part.cells,), sectionName='BeamSection')

# --- Assembly and Boundary Conditions ---

assembly = model.rootAssembly

instance = assembly.Instance(name='BeamInstance', part=part, dependent=ON)

# Fix one end of the beam

end_face = instance.faces.findAt(((beam_length, beam_height / 2.0, beam_width / 2.0),))

end_region = regionToolset.Region(faces=end_face)

model.DisplacementBC(name='FixedEnd', createStepName='Initial', region=end_region, u1=0.0, u2=0.0, u3=0.0)

# --- Apply pressure load on top surface ---

model.StaticStep(name='LoadStep', previous='Initial')

top_face = instance.faces.findAt(((beam_length / 2.0, beam_height, beam_width / 2.0),))

top_surface = assembly.Surface(side1Faces=top_face, name='LoadSurface')

model.Pressure(name='ApplyLoad', createStepName='LoadStep', region=top_surface, magnitude=0.5)

# --- Mesh the part ---

mesh_size = min(hole_diameter, spacing_between_holes) / 2.0

if mesh_size < 25.0:

mesh_size = 25.0

print("[INFO] Meshing with seed size: {}".format(mesh_size))

elem_type = mesh.ElemType(elemCode=C3D8R, elemLibrary=STANDARD)

part.setElementType(regions=(part.cells,), elemTypes=(elem_type,))

part.seedPart(size=mesh_size)

part.generateMesh()

# --- Create and run the simulation job ---

job = mdb.Job(name=job_name, model=model_name)

job.submit()

print("[INFO] Job {} submitted.".format(job_name))

job.waitForCompletion()

print("[INFO] Job {} completed.".format(job_name))

# --- Post-processing results from .odb file ---

odb_path = job_name + '.odb'

if not os.path.exists(odb_path):

print("[WARNING] ODB not found: {}".format(odb_path))

continue

odb = odbAccess.openOdb(path=odb_path)

try:

last_frame = odb.steps['LoadStep'].frames[-1]

except Exception as e:

print("[WARNING] No result frame found: {}".format(e))

odb.close()

continue

stress = last_frame.fieldOutputs['S']

displacement = last_frame.fieldOutputs['U']

# Max stress (Von Mises) and deflection in Y-direction

max_mises = max([v.mises for v in stress.values])

max_u2 = max([abs(v.data[1]) for v in displacement.values])  # vertical displacement

# Safety check

reason = []

if max_mises > yield_strength:

reason.append("Stress")

if max_u2 > max_deflection:

reason.append("Deflection")

status = "YES" if not reason else "NO ({})".format(", ".join(reason))

print("[DEBUG] Max Stress: {:.2f} MPa, Max Deflection: {:.2f} mm → {}".format(max_mises, max_u2, status))

# Add result row

results.append([hole_diameter, hole_spacing, round(max_mises, 2), round(max_u2, 2), status])

odb.close()

# --- Write results to CSV ---

csv_file = 'E:/Abaqus/beam_results.csv'

write_header = not os.path.exists(csv_file)

if results:

with open(csv_file, 'a') as f:

writer = csv.writer(f)

if write_header:

writer.writerow(['Hole Diameter (mm)', 'Hole Spacing (mm)', 'Max Stress (MPa)', 'Max Deflection (mm)', 'Is Safe'])

writer.writerows(results)

print("[INFO] Results written to {}".format(csv_file))

else:

print("[WARNING] No results to write. CSV not created.")

Code for Ga.py(GUI Code) Module

# -*- coding: utf-8 -*-

# GUI for Cellular Beam Safety Simulation using ABAQUS + Genetic Algorithm + Graph Visualization

import tkinter as tk

from tkinter import messagebox

import csv

import subprocess

import os

import matplotlib.pyplot as plt

# --- Save input values to text file ---

def save_input():

try:

diameter = float(entry_diameter.get())

spacing = float(entry_spacing.get())

beam_length = 1000.0

initial_beam_height = 100.0

num_holes = 3

# Safety check: Hole diameter should not exceed beam height limits

if diameter > 0.9 * initial_beam_height:

messagebox.showerror("Invalid Input", f"Hole diameter ({diameter} mm) is too large for the beam height.")

return False

# Check if holes go outside beam length

hole_positions = [i * spacing for i in range(1, num_holes + 1)]

for x in hole_positions:

if x - diameter / 2.0 < 0 or x + diameter / 2.0 > beam_length:

messagebox.showerror("Invalid Input", f"Hole at x = {x:.1f} mm with diameter {diameter} mm would exceed beam boundary.")

return False

# Save to file

with open("beam_input.txt", "w") as f:

f.write(f"{diameter},{spacing}")

messagebox.showinfo("Saved", "Input saved to beam_input.txt")

return True

except ValueError:

messagebox.showerror("Invalid Input", "Please enter valid numeric values.")

return False

# --- Trigger ABAQUS simulation using saved inputs ---

def run_simulation():

if not save_input():

return

try:

messagebox.showinfo("Running", "Simulation is running in ABAQUS...")

subprocess.run(["E:\\Commands\\abaqus.bat", "cae", "noGUI=j.py"], check=True)

messagebox.showinfo("Done", "Simulation completed.")

except Exception as e:

messagebox.showerror("Error", f"Simulation failed:\n{e}")

# --- Load last result from CSV and display it ---

def load_result():

try:

if not os.path.exists("beam_results.csv"):

messagebox.showerror("Missing File", "beam_results.csv not found.")

return

with open("beam_results.csv", "r") as f:

reader = csv.reader(f)

header = next(reader)

rows = [row for row in reader if row and len(row) >= 5]

if not rows:

messagebox.showwarning("No Data", "No valid results found in CSV.")

return

last_row = rows[-1]

safety = last_row[4]

status_color = "green" if safety.strip().upper().startswith("YES") else "red"

result_output.config(fg=status_color)

result_output.config(text=(

f"Safety: {'Safe' if safety.upper().startswith('YES') else 'Unsafe'}\n"

f"Max Stress: {last_row[2]} MPa\n"

f"Max Deflection: {last_row[3]} mm"

))

except Exception as e:

messagebox.showerror("Error", f"Could not read result:\n{e}")

# --- Run Genetic Algorithm Optimization ---

def run_ga():

try:

messagebox.showinfo("Running GA", "Optimization is running. Please wait...")

subprocess.run(["python", "optimize_ga.py"], check=True)

with open("ga_best_result.txt", "r") as f:

result = f.read()

messagebox.showinfo("Best Result from GA", result)

except Exception as e:

messagebox.showerror("GA Error", f"Optimization failed:\n{e}")

# --- Generate and plot graphs from CSV data ---

def plot_graph():

try:

with open("beam_results.csv", "r") as f:

reader = csv.reader(f)

header = next(reader)

rows = []

for row in reader:

try:

if len(row) >= 5:

d = float(row[0])

s = float(row[1])

stress = float(row[2])

defl = float(row[3])

status = row[4].strip().upper()

rows.append((d, s, stress, defl, status))

except:

continue

if not rows:

messagebox.showwarning("No Data", "No valid numeric results to plot.")

return

option = graph_option.get()

plt.figure(figsize=(8, 5))

# 📊 Plot types

if option == "Stress vs Diameter":

sorted_rows = sorted(rows, key=lambda r: r[0])

x = [r[0] for r in sorted_rows]

y = [r[2] for r in sorted_rows]

plt.plot(x, y, marker='o', linestyle='-', color='blue')

plt.xlabel("Hole Diameter (mm)")

plt.ylabel("Max Stress (MPa)")

plt.title("Stress vs Hole Diameter")

elif option == "Deflection vs Diameter":

sorted_rows = sorted(rows, key=lambda r: r[0])

x = [r[0] for r in sorted_rows]

y = [r[3] for r in sorted_rows]

plt.plot(x, y, marker='o', linestyle='-', color='orange')

plt.xlabel("Hole Diameter (mm)")

plt.ylabel("Max Deflection (mm)")

plt.title("Deflection vs Hole Diameter")

elif option == "Stress vs Spacing":

sorted_rows = sorted(rows, key=lambda r: r[1])

x = [r[1] for r in sorted_rows]

y = [r[2] for r in sorted_rows]

plt.plot(x, y, marker='o', linestyle='-', color='purple')

plt.xlabel("Hole Spacing (mm)")

plt.ylabel("Max Stress (MPa)")

plt.title("Stress vs Hole Spacing")

elif option == "Deflection vs Spacing":

sorted_rows = sorted(rows, key=lambda r: r[1])

x = [r[1] for r in sorted_rows]

y = [r[3] for r in sorted_rows]

plt.plot(x, y, marker='o', linestyle='-', color='green')

plt.xlabel("Hole Spacing (mm)")

plt.ylabel("Max Deflection (mm)")

plt.title("Deflection vs Hole Spacing")

elif option == "Safe vs Unsafe":

safe = sum(1 for r in rows if r[4].startswith("YES"))

unsafe = sum(1 for r in rows if r[4].startswith("NO"))

plt.bar(["Safe", "Unsafe"], [safe, unsafe], color=["green", "red"])

plt.ylabel("Number of Configurations")

plt.title("Safety Status Distribution")

plt.grid(True)

plt.tight_layout()

plt.show()

except Exception as e:

messagebox.showerror("Plot Error", f"Could not generate graph:\n{e}")

# ---------------------- GUI Layout ----------------------

# 🌟 Main Window Setup

root = tk.Tk()

root.title("Cellular Beam Safety Simulator with GA")

root.geometry("650x700")

root.configure(bg="#f7f7f9")

root.resizable(False, False)

# 🔷 Title

title_label = tk.Label(root, text="Cellular Beam Safety Simulator", font=("Helvetica", 20, "bold"), fg="#222", bg="#f7f7f9")

title_label.pack(pady=(20, 10))

# 📥 Input Section

input_frame = tk.LabelFrame(root, text="Input Parameters", font=("Helvetica", 13, "bold"), bg="white", fg="#444", padx=20, pady=15)

input_frame.pack(padx=25, pady=10, fill="x")

tk.Label(input_frame, text="Hole Diameter (mm):", font=("Helvetica", 12), bg="white").grid(row=0, column=0, sticky="e", pady=5)

entry_diameter = tk.Entry(input_frame, font=("Helvetica", 12), width=30)

entry_diameter.grid(row=0, column=1, pady=5, padx=10)

tk.Label(input_frame, text="Hole Spacing (mm):", font=("Helvetica", 12), bg="white").grid(row=1, column=0, sticky="e", pady=5)

entry_spacing = tk.Entry(input_frame, font=("Helvetica", 12), width=30)

entry_spacing.grid(row=1, column=1, pady=5, padx=10)

# 🎛️ Control Buttons

action_frame = tk.LabelFrame(root, text="Controls", font=("Helvetica", 13, "bold"), bg="white", fg="#444", padx=20, pady=15)

action_frame.pack(padx=25, pady=10, fill="x")

tk.Button(action_frame, text="Run Simulation", font=("Helvetica", 12, "bold"), bg="#007bff", fg="white", width=20, command=run_simulation).grid(row=0, column=0, padx=10, pady=5)

tk.Button(action_frame, text="View Result", font=("Helvetica", 12, "bold"), bg="#28a745", fg="white", width=20, command=load_result).grid(row=0, column=1, padx=10, pady=5)

tk.Button(action_frame, text="Run GA Optimization", font=("Helvetica", 12, "bold"), bg="#ff8c00", fg="white", width=42, command=run_ga).grid(row=1, column=0, columnspan=2, pady=5)

# 📈 Graph Section

graph_frame = tk.LabelFrame(root, text="Graph Visualization", font=("Helvetica", 13, "bold"), bg="white", fg="#444", padx=20, pady=15)

graph_frame.pack(padx=25, pady=10, fill="x")

tk.Label(graph_frame, text="Select Graph Type:", font=("Helvetica", 12), bg="white").grid(row=0, column=0, sticky="e", pady=5)

graph_option = tk.StringVar()

graph_option.set("Stress vs Diameter")

graph_menu = tk.OptionMenu(graph_frame, graph_option,

"Stress vs Diameter",

"Deflection vs Diameter",

"Stress vs Spacing",

"Deflection vs Spacing",

"Safe vs Unsafe"

)

graph_menu.config(font=("Helvetica", 11))

graph_menu.grid(row=0, column=1, pady=5, padx=10)

tk.Button(graph_frame, text="Plot Graph", font=("Helvetica", 12, "bold"), bg="#6f42c1", fg="white", width=42, command=plot_graph).grid(row=1, column=0, columnspan=2, pady=10)

# 📋 Output Result Box

result_label = tk.Label(root, text="Simulation Result", font=("Helvetica", 14, "bold"), bg="#f7f7f9")

result_label.pack(pady=(10, 5))

result_output = tk.Label(root, text="", font=("Helvetica", 12), fg="black", bg="white",

justify="left", relief="solid", bd=2, width=65, height=6, anchor="nw")

result_output.pack(padx=20, pady=10)

# 🌐 Launch GUI

root.mainloop()

Code for optimization with ga

import random

import subprocess

import csv

import os

from pyeasyga.pyeasyga import GeneticAlgorithm

# Fitness function: run j.py with given inputs and read results

def fitness_func(individual, data=None):

diameter, spacing = individual

# Write to beam_input.txt

with open("beam_input.txt", "w") as f:

f.write(f"{diameter},{spacing}")

# Run ABAQUS simulation

try:

subprocess.run(["E:\\Commands\\abaqus.bat", "cae", "noGUI=j.py"], check=True)

except Exception as e:

print(f"[ERROR] ABAQUS failed for D={diameter}, S={spacing}: {e}")

return 0  # Penalize failed simulations

# Read results from CSV

try:

with open("E:/Abaqus/beam_results.csv", "r") as f:

rows = list(csv.reader(f))

if len(rows) < 2:

return 0

last = rows[-1]

stress = float(last[2])

deflection = float(last[3])

is_safe = last[4].strip()

if is_safe.upper() != "YES":

return 0  # Penalize unsafe designs

return 1 / (deflection + 1e-6)  # Lower deflection = higher score

except Exception as e:

print(f"[ERROR] Failed to read results: {e}")

return 0

# Configure the genetic algorithm

ga = GeneticAlgorithm(

seed_data=None,

population_size=6,

generations=10,

crossover_probability=0.8,

mutation_probability=0.2,

elitism=True

)

ga.fitness_function = fitness_func

ga.create_individual = lambda data: [random.randint(50, 100), random.randint(200, 500)]

ga.run()

# Output the best result

best = ga.best_individual()

print(f"Best Config: Diameter = {best[1][0]}, Spacing = {best[1][1]}")

print(f"Fitness Score: {best[0]}")

# Save best result to file

with open("ga_best_result.txt", "w") as f:

f.write(f"Best Diameter: {best[1][0]}\n")

f.write(f"Best Spacing: {best[1][1]}\n")

f.write(f"Fitness Score: {best[0]}\n")

  - User Manual

### 1. Overview

- This system automates the modelling and structural analysis of cellular steel beams using ABAQUS and Python. Users can enter design parameters, simulate stress and deflection, view safety results via a GUI, and perform optimization using Genetic Algorithms (GA).

### 2. Intended Users

- Student Users – Engineering students conducting structural analysis projects.

- Professional Users – Engineers exploring safe beam configurations efficiently.

- Research Users – Academics optimizing design parameters using GA.

- Instructor Users – Educators demonstrating FEA and automation in class.

### 3. System Requirements

- ABAQUS Student Edition 2021 or compatible version

- Python 2.7 – for running j.py with ABAQUS

- Python 3.x – for GUI and GA interfaces

- Recommended OS: Windows

### 4. Folder Setup

- Ensure all files are in one working folder:

- ga.py – Graphical User Interface (Python 3) for parameter input and result display

- j.py – ABAQUS automation script (Python 2.7) for modelling and simulation

- optimization_with_ga.py – Genetic Algorithm script to find optimal beam design

- beam_input.txt – Stores input values from the GUI

- beam_results.csv – Stores output values (stress, deflection, verdict)

### 5. Running the System

#### Step 1: Launch the GUI

- Open Command Prompt / Terminal

- Navigate to the folder:

cd path/to/your/folder

- Run the GUI:

python ga.py

#### Step 2: Use the Interface

- Enter hole diameter and hole spacing.

- Click "Save Input" to store values.

- Click "Run Simulation" to call j.py and run ABAQUS.

- Click "Load Result" to see:

  - Maximum Von Mises Stress

  - Maximum Vertical Deflection

  - Safety Status (SAFE/UNSAFE)

#### Step 3: View Graphs

- Click plot graph, choose the type of graph you want to plot from the dropdown

### 6. Running Genetic Algorithm Optimization

#### Step 1: Launch the Optimizer

python optimization_with_ga.py

#### Step 2: What it Does

- Generates a range of random hole diameter and spacing values

- For each design, calls j.py to simulate in ABAQUS

- Evaluates based on fitness (low deflection + safe stress)

- Applies GA logic: selection, crossover, and mutation

- Returns the most optimal and safe beam configuration

CODE BOOKLET

SCHOOL OF COMPUTING

2022

CB_V1_2022